- Instruction format (Assembly Code)

[ 4-bit opcode | 3-bit Rd | 3-bit Rs | 6-bit immediate ]

- Opcode Table

| Opcode | Mnemonic | Description |
| --- | --- | --- |
| 0000 | MOV | Loads an intermediate value into a register Nothing is read from memory. The value comes straight from the instruction itself. |
| 0001 | LOAD | Reads a value from a memory address into a register. Used to read from RAM or MMIO peripherals like buttons and switches. |
| 0010 | STORE | Writes a value from a register into a memory address. Used to write to RAM or MMIO peripherals like LEDs, 7-seg, VGA. |
| 0011 | ADD | Adds two registers together, stores result in destination. Used in your lock program to increment the attempt counter. |
| 0100 | SUB | Subtracts one register from another. |
| 0101 | AND | Bitwise AND between two registers. Useful for masking specific bits. |
| 0110 | OR | Bitwise OR between two registers. Useful for setting specific bits. |
| 0111 | CMP | Subtracts two registers and sets flags but throws away the result. Used right before a conditional jump. This is how your CPU checks if the entered code matches the passcode. |
| 1000 | JMP | Unconditional jump to an address. Always taken. Used for loops that never exit on their own. |
| 1001 | JZ | Conditional jump. Only jumps if the Z flag = 1 (last result was zero). Since CMP sets Z=1 when two values are equal, JZ is how you check equality. |
| 1010 | JNZ | Conditional jump. Only jumps if the Z flag = 0 (last result was not zero. Opposite of JZ. |
| 1011 | JN | Conditional jump. Only jumps if the N flag = 1 (last result was negative). Used when you need to check if one value is less than another. |
| 1100 | NOP | Does absolutely nothing for one clock cycle. Useful for timing delays or as a placeholder while writing code. |
| 1101 | HALT | Stops the CPU completely. Used at the end of your lockout sequence. Once locked out, the CPU stops until the system is reset. |

| Opcode | Mnemonic | Rd | Rs | Immediate |
| --- | --- | --- | --- | --- |
| 0000 | MOV | Destination = 0 | Unused | Value (0-63) = 00 |
| 0001 | LOAD | Destination | Unused | Memory Address = 01 |
| 0010 | STORE | Source | Unused | Memory Address = 01 |
| 0011 | ADD | Destination | Source = 0 | Unused |
| 0100 | SUB | Destination | source | Unused |
| 0101 | AND | Destination | source | Unused |
| 0110 | OR | Destination | source | Unused |
| 0111 | CMP | Operand 1 | Operand 2 | Unused |
| 1000 | JMP | Jump Address = 1 | Jump Address = 1 | Jump Address = 10 |
| 1001 | JZ | Jump Address | Jump Address | Jump Address = 10 |
| 1010 | JNZ | Jump Address | Jump Address | Jump Address = 10 |
| 1011 | JN | Jump Address | Jump Address | Jump Address = 10 |
| 1100 | NOP | Unused | Unused | Unused |
| 1101 | HALT | Unused | Unused | Unused |

Decoders

Immediate (Value 0-63) = A’B’C’D’ = 00

Immediate (Memory Address) = A’B’C’D + A’B’CD’ = A’B’(C’D + CD’) = 01

Immediate (Jump Address) = AB’C’D’ + AB’C’D + AB’CD’ + AB’CD = AB’ = 10

Rs (Source Register) = A’B’CD + A’BC’D’ + A’BC’D + A’BCD’ + A’BCD = A’(B + CD) = 0

Rs (Jump Address) = AB’C’D’ + AB’C’D + AB’CD’ + AB’CD = AB’ = 1

Rd (Destination Register) = A’B’C’D’ + A’B’C’D + A’B’CD’ + A’B’CD + A’BC’D’ + A’BC’D + A’BCD’ + A’BCD = A’ = 0

Rd (Jump Address) = AB’C’D’ + AB’C’D + AB’CD’ + AB’CD = AB’ = 1

Encoders

4 line to 2 line Encoder

| D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 | A | B | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |

| Instruction | Condition | Flag Checked | When Taken |
| --- | --- | --- | --- |
| JMP | None | None | Always |
| JZ | Equal/Zero | Z=1 | Values were equal |
| JNZ | Not Equal/ Not Zero | Z=0 | Values were not equal |
| JN | Negative | N=1 | Result was negative |

1-bit Comparator Truth Table Reference

| A | B | A = B | A > B | A < B |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |

- Register Table

| Register | Binary |  |
| --- | --- | --- |
| R0 | 000 | Zero Constant (Always 0, used for comparisons |
| R1 | 001 | Attempt Counter (0, 1, 2, 3) |
| R2 | 010 | Stored Passcode (The correct code. E.g. 5) |
| R3 | 011 | Button Input (Read from 0x8001) |
| R4 | 100 | Switch Input (Read from 0x8000, entered code) |
| R5 | 101 | VGA Status Code (0,1,2,3,4 written to 0x8004) |
| R6 | 110 | Lockout Threshold (Constant 3, for comparison) |
| R7 | 111 | Spare / Temp (Just in case) |

Read-Only Peripheral Register Addresses

BT_Reg = 110100

SW_Reg = 110101

Write-Only Peripheral Registers Addresses

LED_Reg = 110110

RGB_Reg = 110111

Seg 7 = 111000

Seg 6 = 111001

Seg 5 = 111010

Seg 4 = 111011

Seg 3 = 111100

Seg 2 = 111101

Seg 1 = 111110

Seg 0 = 111111

Ram Memory Addresses

| Dec | B5 | B4 | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- |